- CHC EasyNav has several useful features. Of the following which one is a feature.

  - Ability to Import a design

  - Load point data collected with a rover

  - Upgradable system

- The system has two antennas installed on the boom of the machine. Which antenna is considered the main antenna?

  - Front

  - Back

- The system has two GNSS antenna connection ports on the back of the display. Which antenna port is for the Aux Antenna.

- The bucket sensor can be installed on which of the following (Choose all that apply)

  - Bucket

  - Dog bone

  - Quick Coupler

  - Bucket Linkage

- When pulling measurements for the GNSS antennas. The measurement should be based on where on the antenna?

  - The reference mark on the bracket

  - Back of bracket

  - Outside edge of antenna

- A framing square is needed for the calibration. Which routine is the square used for?

  - Measuring the boom thickness

  - Used to measure the stick length

  - Utilized for the O-M and O-P for the stick pin

- When conducting training what is the first thing you should show a customer?

  - How to create a surface design

  - Connecting to a radio

  - How to change buckets

  - Screen Layout (User Interface)

- What is the remote support application on the system called?

  - Mobi Control

  - Quick Support

  - SiteLink

  - Google Chrome

- Will the EasyNav system work on a swing boom machine without additional hardware?

  - True

  - False

- The recommended install and calibration time is?

  - 8 Hours

  - 16 Hours

  - 4 Hours

  - None of the above

- The kit provides weld on brackets. Where is the proper location to weld while installing the weld on brackets?

  - Top and bottom of bracket

  - Left and Right Side of the bracket

  - Center holes on bracket

  - All the above

- When determining the location for the display it is important to check for the following?

  - Front glass clearance

  - Operators view from seat

  - OEM gauge clearance

  - All of the above

- When installing the main GNSS antenna it is best to install the bracket?

  - Within 1 m of the stick pin towards the bottom of the boom

  - Within 1 m of the stick pin towards the top of the boom

  - On the stick pin

  - None of the above

- After calibration a flat to vertical bucket check is performed. What does this check confirm?

  - The bucket is calibrated correctly

  - The Stick is properly calibrated

  - All hardware is installed properly

  - None of the above

- The system can not be calibrated on the following unit of measurements?

  - US Survey Feet 0.01’

  - Meters

  - Feet and Inches

- When calibrating the bucket sensor, if the sensor is installed on the quick coupler. The tech must ensure the dog bone is horizontal before calibrating?

  - True

  - False

- All measurements based on the pins are measured to the_____ of the pin?

- What are the two types of power connection available for the system?

- All the sensors including the GNSS antennas must be installed on the same side of the boom?

  - True

  - False

- When align the sensor before welding they should be______ to the centerline of the pins?

  - Parallel

  - Perpendicular

  - Does not matter

- While installing the GNSS antennas it is important for the connection port to be facing in which direction?

  - Connector towards machine cab

  - Connector towards bucket

  - Connector towards boom facing inwards

- Each sensor has an arrow indicating the direction the sensor should be installed. Which is the correct orientation of the arrow

  - Towards the Cab

  - Facing upperwards

  - Facing downwards

  - Towards the bucket

- What are the two connection types that give you a fixed position in the system?

  - Radio and Surface

  - Surface and Network(CORS)

  - Radio and Network(CORS)

- There is more than one way to view recorded points within the system

  - True

  - False

- What is the purpose of the bucket focus point button?

  - It shows the size of the bucket

  - It shows which bucket tooth will read grade

  - It changes coordinate systems

  - All of the above

- You can not edit a surface after you have created it

  - True

  - False

- You can create a surface from both the bucket teeth or a recorded point?

  - True

  - False

- The purpose of the Coordinate system is?

- Place a machine within a certain state boundary

- Change buckets

- Broadcast a frequency

- None of the above

- When setting up a radio connection for the machine what settings icon is it located within?

- System

- Datalink

- About

- Point management

Please email the completed test to remoteInstall@benchmarksupply.com, please include your contact information in the email body and with your availability to follow up with Benchmark to discuss before proceeding with installation.